Name:vrajesh

Question 1:

Draw a diagram of the OSI 7-layer model and label each layer with its correct name and a real-world example of a device or protocol that operates at that layer (e.g., Switch at Data Link, HTTP at Application).

Answer:

+------------------------+

| 7. Application Layer   | HTTP, HTTPS, FTP

+------------------------+

| 6. Presentation Layer  | SSL/TLS Encryption

+------------------------+

| 5. Session Layer       | NetBIOS, RPC

+------------------------+

| 4. Transport Layer     | TCP, UDP

+------------------------+

| 3. Network Layer       | Router, IP

+------------------------+

| 2. Data Link Layer     | Switch, MAC Address

+------------------------+

| 1. Physical Layer      | Cable, Hub

+------------------------+

Question 2:

Match the following networking scenarios to the correct OSI layer:

a) Encrypting data before sending

b) Routing packets across networks

c) Converting data to electrical signals

d) Opening a web page in your browser

e) Detecting and correcting errors in frames

Hint: Write your answers as 'Scenario: Layer Name (Layer Number)'.

Answer:

a) Encrypting data before sending: Presentation Layer (Layer 6)

b) Routing packets across networks: Network Layer (Layer 3)

c) Converting data to electrical signals: Physical Layer (Layer 1)

d) Opening a web page in your browser: Application Layer (Layer 7)

e) Detecting and correcting errors in frames: Data Link Layer (Layer 2)

Question 3:

Pick any app you use daily (like WhatsApp, Instagram, or Zomato) and describe how at least three different OSI layers are involved when you send or receive data in that app.

Answer (WhatsApp Example):

When I send a message on WhatsApp:

Application Layer (Layer 7) – WhatsApp creates and displays the message.

Transport Layer (Layer 4) – TCP ensures the message is delivered correctly.

Network Layer (Layer 3) – IP addresses help route the message through the internet.

Data Link Layer (Layer 2) – Wi-Fi or mobile network transfers the data.

Physical Layer (Layer 1) – Data travels as radio waves or electrical signals.

Question 4:

Given this scenario: You are unable to access a website, but you can successfully ping its IP address. Identify which OSI layer(s) might be causing the problem and explain your reasoning.

Answer:

The problem is likely in the Application Layer (Layer 7), Presentation Layer (Layer 6), or Session Layer (Layer 5).

Reason:

Ping works, which means the Network Layer (Layer 3) is functioning correctly.

The device can reach the server's IP address.

The issue may be due to:

DNS problems

HTTP/HTTPS service not working

SSL/TLS certificate issues

Browser configuration problems

Conclusion: Since the network connection is working, the problem is most likely in Layer 5, Layer 6, or Layer 7.